CHAPTER 9

Opening the transceiver's case

Follow the case and cover opening procedures shown here when you want to install an optional unit or adjust an internal unit, etc.

CAUTION: DISCONNECT the DC power cable from the IC-718 before performing any work on the transceiver. Otherwise, there is danger of electric shock and/or equipment damage.

Remove the 5 screws from the top of the transceiver and 4 screws from the sides, then lift up the top cover.

Remove the 5 screws from the bottom of the transceiver, then remove the bottom cover.

Optional bracket and carrying handle

Mounting bracket

An optional IC-MB5 MOBILE MOUNTING BRACKET is available to install the radio under a table, on a wall, in a vehicle, etc. Select

an area to mount the receiver keeping in mind that the weight of the transceiver is approx. 3.80kg.

Carrying handle

An optional handle allows you to easily carry and

transport the transceiver.

Attach the MB-23 CARRYING HANDLE with the supplied rubber feet as shown.

CR-338 HIGH STABILITY CRYSTAL UNIT

By installing the CR-33, the total frequency stability of the receiver will be improved.

Remove the bottom cover as shown in the diagram before.

Disconnect W2 from J4401 (MAIN unit) and W3 from J4201 (MAIN unit).

Remove 9 screws from the PLL unit, disconnect P4 from J201 (MAIN unit) and P2 from J401 (MAIN unit), then remove the PLL unit.

Remove the supplied internal crystal and replace with the CR-338.

Return the PLL unit, plugs and flat cables to their original positions.

Adjust the reference frequency at C16 using a frequency counter if desired.

• Connect the frequency counter to P. 2 (PLL unit).

Return the bottom cover to its original position.

UT-102 VOICE SYNTHESIZER UNIT

The UT-102 announces the received frequency, mode, S-meter level and current time in a clear, electronically-generated voice, in English (or Japanese). Push [LOCK] for 1 sec. to announce the frequency,  etc.

Remove the bottom cover as shown above

Remove the protective paper attached to the bottom of the UT-102 to expose the adhesive strip.

Plug UT-102 into J2501 on the MAIN unit as shown at right.

Return the bottom cover to its original position.

• UT-106 DSP RECEIVE UNIT

The UT-106 provides AF DSP functions such as noise reduction and auto notch.

Remove the bottom cover.

Slide the insulating case onto the UT-106

as shown right. (Fig. 1)

Remove the connection cable (P2601) from J2602 on the MAIN unit. Connect the cable into J1 on the UT-106.

Plug the connection cable (P1) from the UT-106 to J2602 on the MAIN unit.

Plug the flat cable into J3 on the UT-106 and to J2603 on the MAIN unit.

• Take care regarding the conductor direction.

Turn the UT-106 unit over. (Fig. 2)

• No need to fix with an adhesive strip, etc.

Put the UT-106 on the MAIN unit.

•	No need to fix with an adhesive strip, etc.

•	Ensure that the surplus cable from UT-106 is stored under the unit.

Return the bottom cover to its original position.

• Optional IF filters

Several IF filters are available for the IC-718. You can install 1 filter for 455 KHz IF. Choose the appropriate filter for your operating needs. (pgs. 24-25)

Installation

Remove the bottom cover as shown on the p. 48.

Remove 7 screws, connection cable p1 from J1, p5 from J701, W4 from J4101 and W5 from J4001 and 2 Tr-clampers as shown in the diagram below.

Install the desired 455 KHz filter as shown in the diagram below.

Mounting the filter with supplied washers and nuts.

Solder the 4 leads.

Return the MAIN unit and bottom cover to their original positions.

After filter installation, specify the installed filter using initial set mode. (p. 47) Otherwise, the installed filter will not function properly.

AT-180 internal switch description

The optional AT-180 has 3 operating conditions for HF band operation. Select a suitable condition according to your antenna system.

Remove the top cover of the AT-180,

Set the tuner switches to the desired positions

according to the table below.

SW        Position                                        Operation,

A                              The tuner operating condition is set 
		       (default)                                by S2     described below.

THROUGH INHIBIT
				                            The tuner tunes the antenna even when
				                            the antenna has poor SWR (up to VSWR
S1				                           3:1 after tuning). In this case, manual tun-
	    			                    ing is necessary each time you change the

B	                       frequency although the tuner automatically
			                      starts tuning when the VSWR is higher
			                      than 3:1. This setting is called "through
			                     inhibit," however, the tuner is set to
			                    "through" if the VSWR is higher than 3:1
			                     after tuning.

----------------------------------------------------------------------------------------------------
			                  TUNER SENSITIVE CONDITION
			                The tuner tunes each time you transmit
		C	                (except SSB mode). Therefore, the lowest
			                  SWR Is obtained at any given time.
			                 For SSB mode, the same condition is as
			                 the "D" position below.

S2
			                NORMAL CONDITION
		D	             The tuner tunes when the SWR is higher

(default).	          than 1.5:1. Therefore, the tuner activates only when tuning is necessary.

• Specifications for the AT180

• Frequency coverage : 1.9-54 MHz

• Input impedance	: 50 ohm
• Maximum input	: 120 W
power
• Minimum tuning	: 8 W

power

• Matching impedance : 16.7-150  ohm(HF band)

range                          :20-125 ohm (50 MHz band)

Tuning accuracy           :Less than SWR 1.5:1

• Insertion loss:		:Less than 1.0 dB  (after tuning)

•	Power supply requirements	:13.8 V DC/1 A (supplied from the transceiver's ACC socket)

•	Dimensions (mm/in) :	167(W) x58.6(H) x225(D)

6  9/16(W) x 2  5/17(H) x 8 7/8 (D)

Weight	:	2.4 kg; 5 Ib 4 oz

• Supplied accessories: coaxial cable (1 m),

ACC cable (DIN 13 pins)

Chapter 10:   MAINTENANCE

• Troubleshooting

The following chart is designed to help you correct problems which are equipment malfunctions.

If you are not able to locate the cause of a problem or solve it through the use of this chart, contact your nearest  From  Dealer or Service Center.

Power does not come on when the [POWER] switch is pushed.

•	DC power cable is improperly connected.

• Reconnect the DC power cable correctly.            P. 12

•	Fuse is blown.

• Check for the cause, then replace the fuse with a spare one.       p.53

(Fuses are installed in the DC power cable and the internal PA unit.)

•	Power Supply not turned ON.

No sound comes from the speaker.

•	Volume level is too low.

• Rotate [AF] clockwise to obtain a suitable	p. 2
listening level.

•	The squelch is closed.

• Rotate [RF/SQL] to 10 o'clock position to open	p. 2
the squelch.

•	The transceiver is in the transmitting condition.

• Check the SEND line of an external unit, if	p. 6
desired.

Sensitivity is low.

•	The antenna is not connected properly.

• Reconnect to the antenna connector.	--

•	The antenna for another band is connected.

• Connect an antenna suitable for the	--
operating frequency.

• The antenna is not properly tuned.

• Push [TUNER] for 2 sec. to manually tune the	p. 3
antenna.

• The attenuator is activated.
• Push [ATT] to select "ATT" OFF.	p. 3

Receive audio is distorted.

•	The operating mode is not selected correctly.

Select a suitable operating mode.	p. 20

•	IF SHIFT function is activated.

• • Rotate the SHIFT control to center position,	p. 21

•	Noise blanker function is activated.

• Push [NB] to turn the function OFF.	p. 21

•	Preamp is activated.

• Push [P.AMP] to turn the function OFF.	p. 22

•	The noise reduction is activated and the [NR] control is set too high.

• Set the [NR] control for maximum readability,    p. 23

Transmitting is impossible.

•The operating frequency is not set to a ham  band.

• Set the frequency to a ham band.  P. 17

Output power is too low.

• [RF POWER] is set too low.
• Set [RF POWER] to a suitable position,	p. 42

[MIC GAIN] is set too low.
• Set [MIC GAIN] to a suitable position,	p. 42

• The antenna for another band is selected.

• Select an antenna suitable for the operating	p. 10

frequency.

• The antenna is not properly tuned.

•Push [TUNER] for 2 sec. to manually tune the antenna.  P. 3

No contact possible with another station.

•	RIT function is activated.

• Push [RIT] to turn the function OFE	p. 21

•	Split frequency function is activated.

• Push [SPLIT] to turn the function OFF.	pgs. 7
	31,32

Transmitted signals are distorted.

• [MIC GAIN] is set too high.	
• Set [MIC GAIN] to a suitable position,	p. 2

• [COMP] function is activated.

• Turn [COMP] off.	 p. 27

Programmed scan does not stop.

• Squelch is open.

• Set [RF/SQL] to the threshold point,	p. 3

• [RF/SQL] is assigned to RF gain control  and  squelch is open.

• Reset [RF/SQL] control assignment and set it to the threshold point.

p. 30

Programmed scan does not start.

The same frequencies have been programmed		
in scan edge memory channels P1 and P2.

Program different frequencies in scan edge

memory channels P1 and P2.     p. 40

Memory scan does not start.

• 2 or more memory channels have not been programmed.

Program 2 or more memorychannels,	p. 40

The displayed frequency does not change properly.

• The dial lock function is activated.

• Push [LOCK] to deactivate the function,	p. 6

• A quick set mode screen is selected.

Push [SET] to exit the quick set mode.	p. 41

• The internal CPU has malfunctioned.

• Reset the CPU.	 p. 53

Fuse replacement

If a fuse blows or the transceiver stops functioning, try to find the source of the problem, and replace the damaged fuse with a new, rated fuse.

CAUTION: DISCONNECT the DC power cable

from the transceiver when changing a fuse.	•

The IC-718 has 2 types of fuses installed for transceiver protection.

• DC power cable fuses 	FGB 20 A
• Circuitry fuse 	FGB 4 A

CIRCUITRY FUSE REPLACEMENT

The 13.8 V DC from the DC power cable is applied to all units in the IC-718 through the circuitry fuse. This fuse is installed In the MAIN unit.

Remove the top cover as shown on p. 48 Replace the circuitry fuse as shown In the diagram at right.

Replace the top cover.

Resetting the CPU

Resetting CLEARS all programmed contents in  memory channels and returns programmed values

in set mode to their defaults.

When first applying power or when the function seems to be displaying erroneous information, reset the CPU as follows:

Make sure transceiver power is OFF.

While pushing [UP] and I DN], push [PWR] to turn power ON.

•	The internal CPU is reset.

•	The transceiver displays its initial VFO frequencies when resetting is complete.

Chapter 11:  SPECIFICATIONS

General

• Frequency coverage

Receive 	0.03-29.999999 MHz *1

Transmit	1.800 - 1.999999 MHz.*2

2 3.500 - 3.999999 MHz.*2

2 7.000 - 7.300000 MHz

10.100 - 10.150000 MH

14.000 - 14.350000 MHz

18.068 - 18.168000 MHz

21.000 - 21.450000 MHz

24.890  -24.990000 MHz

28.000 - 29.700000 MHz

*1  Guaranteed range: 0.5-29.999999 MHz

*2  Varies according to version

• Mode			: USB, LSB, CW, RTTY, AM,

•	Number of memory channels       : 101 (99 regular, 2 scan edges)

• Frequency stability 	:Less than +-200 Hz from 1 min. to 60 min. after power on. After that rate of stability less than +-30 Hz/hr. at +25°C (+77°F). Temperature fluctuations 0°C to +50°C (+32°F to +122°F) less than +-350 Hz.

•	Power supply requirement		: 13.8V DC  +-15% (negative ground)

•	Current drain (at 13.8 V DC) :

: Receive

Standby 1.3 A

Max.audio 2.0 A

Transmit

Max. power 20.0A

*Operatable temp. range:	-10°C to +60°C; +14°F to +140°F

• Antenna connector	: SO-239 (50 ohm)

•	Dimensions : 240(W) x 95(H) x 239(D) mm (projections not included)

97  7/8(W) x 3  3/4(H) x 9  13/32 (D) in

• Weight (approx.)	: 3.8 kg; 8 Ib 6 oz
• ACC connector	: 13pin
• REMOTE connector	: 2-conductor 3.5 (d) mm (1/8')

Transmitter

• Output power

SSB, CW, RTTY          2-100 W

AM			  2-40 W

• Modulation system

SSB		 Balanced modulation

AM		Low level modulation

• Spurious emissions	: Less than -50 dB below  peak output power

*Spurious freq.; Below 30 MHz: -50 dB

Above 50 MHz: -60 dB

NOTE: Limit in dB decreases linearly with thw logarithm of frequency in the range 30 MHz to 50 MHz.

•	Carrier suppression : More than 40 dB

•	Unwanted sideband : More than 50 dB

•	Microphone connector : 8-pin connector (600 ohm)

• Key connector	: 3-conductor 6.5 (d) mm (1/4")

SEND/ALC connector : Phono (RCA)

Receiver

• Receive system               Double-conversion superheterodyne system

• Sensitivity

SSB, CW, RTTY		0.16 uV (1.8-29.999999 MHz)

AM				13 uV (0.5-1.799999 MHz)

2 uV (1.8-29.999999 MHz)

• Squelch sensitivity (threshold):

SSB, CW, RTTY Less than 5.6 uV

• Selectivity

SSB, CW, RTTY         More than 2.1 kHz/-6 dB

Less than 4.5 kHz/-60 dB

AM                            More than 6 kHz/-6 dB

Less than 20 kHz/-40 dB

• Spurious and image rejection ratio:

More than 70 dB (1.8-29.999999 MHz)

• RIT variable range: +-1200 Hz

•	Audio output power (at 13.8 V DC) :	More than 2.0 W at 10% distortion with an 8 ohm load

• PHONES connector:              3-conductor 6.35 (d) mm  1/4 "

• External SP connector: 2-conductor 3.5 (d) mm     1/8 "  8 ohm

All stated specifications are typical and subject to change without notice or obligation.