CHAPTER 12  OPTIONS

IC-PW1 HF + 50 MHz 1 KW LINER AMPLIFIER

Full-duty 1 kW linear amplifier  including an automatic antenna tuner. Has automatic tuning and band selection capability. Full break-in (QSK) operation is possible. The amplifier/power supply unit and the remote contro lunit are separated.

AT-180 HF + 50 MHz AUTOMATIC ANTENNA TUNER

Fully automatic antenna tuner with preset memories for each 100 KHz. Unique "Automatic tuner on " function is available. See P. ?? for AT- 180 specifications.

AH-4 HF + 50 MHz AUTOMATIC ANTENNA TUNER

Specially designed to tune a long wire antenna for portable or mobile HF operation.  The PTT tuner start function provides simple operation.

• Input power rating: 120 W

AH-2b ANTENNA ELEMENT

A 2.5 m long antennaelement for mobile operation with the AH4.

• Frequency coverage:

3.5-28 MHz bands with the AH-4

PS-85 DC POWER SUPPLY

Light weight switching regulator system

power supply.

• Output voltage: 13.8 V DC

• Max. current drain: 20 A

SM-8 DESKTOP MICROPHONE

Including 2 connection cables for simultaneous connection of 2 transceivers.  Has [UP]/[DOWN] switches. The OPC-589 is necessary to use microphone.

SM-20 DESKTOP MICROPHONE

Unidirectional, electret microphone for base station operation.  Includes [UP]/[DOWN] switches and a low cut function.

SM-6 DESKTOP MICROPHONE

Electret condenser-type desktop microphone.

HM-36 HAND MICROPHONE

Hand microphone equipped with

[UP]/[DOWN] switches.

Same as supplied.

SP-2OEXTERNALSPEAKER

4 audio filters; headphone jack; can

connect to 2 transceivers.

• Input impedance: 8 ohm

• Max. imput power: 5 W

SP-21EXTERNALSPEAKER

Designed for base station operation.

Input impedance: 8 ohm

Max. input power: 5 W

SP-7  EXTERNALSPEAKER

Compact speaker for base station operation.  Height can be adjusted for your convenience.

FL-52A, FL-53A, FL-96, FL-222 and FL-257 455 KHz FILTERS

•	FL-52A: 500 Hz/-6dB (CW/RTTY narrow)

•	FL-53A: 250 Hz/-6dB (CW narrow)

•	FL-96:2.8 KHz/-6dB (SSB wide)

•	FL-222:1.8 KHz/-6dB (SSB narrow)

•	FL-257:3.3 KHz/-6dB (SSB wide)

UT-106 DSP RECEIVE UNIT

Provides AF DSP functions such as noise reduction and auto notch.

UT-102 VOICE SYNTHESIZER

Announces the receive frequency, mode, S-meter level and current time in a clear, electronically-generated voice, in English (or Japanese).

CR-338	HIGH-STABILITY CRYSTAL  UNIT

Contains a temperature-compensating oven heater and crytal unit for improved frequency stability.

• Frequency stability: +-0.5 ppm

MB-23 CARRYING HANDLE

Carrying handle, convenient for portable operation.

IC-MB5	MOBILE MOUNTING BRACKET

Transceiver mounting bracket for mobile   operation.

CT-17 CI-V LEVEL CONVERTER

For remote receiver control using a personal computer. You can change frequencies, operating mode, memory channels, etc.

AH-710	FOLDED DIPOLE ANTENNA

Covers from 1.9-30 MHz bands. Has an SO-239 connector. 30 m (98.4 ft) coaxial cable with PL-259 connector is supplied.

OPC-599 ADAPTER CABLE

13-pin, ACC connector to 7-pin + 8-pin ACC connector.

CHAPTER 13:  CONTROL COMMAND

• Remote jack (CI-V)information

oCI-V connection example

The transceiver can be connected through an optional CT-17 CI-V LEVEL CONVERTER to a personal computer equipped with an RS-232C port. The Icom Communications Interface-V (CI-V) controls the following functions of the receiver.

Up to 4 Icom CI-V transceivers or receivers can be connected to a personal computer equipped with an RS-232C port. See p. 32 for setting the CI-V condition using set mode.

Data format

The CI-V system can be operated using the following data formats.   Data formats differ according to command numbers.  A data area or sub command is added for some commands,

BC-25