CHAPTER 3

INSTALLATION AND CONNECTIONS

• Unpacking

After unpacking, immediately report any damage to the delivering carrier or dealer. Keep the shipping cartons.

For a description and a diagram of accessory equipmentincluded with the IC-718, see 'Supplied accessories'on p. 1 of this manual.

• Selecting a location

Select a location for the transceiver that allows adequateair circulation, free from extreme heat, cold, or vibrations, and away from TV sets, TV antenna elements radios and other electro-magnetic sources.

The base of the transceiver has an adjustable stand for desktop use. Set the stand to one of two angles dependingon your operating conditions.

• Grounding

To prevent electrical shock, television interference (TVI), broadcast interference (BCI) and other problems, ground the transceiver through the GROUND terminal on the rear panel.

For best results, connect a heavy gauge wire or strap to a long earth-sunk copper rod. Make the distance between the [GND] terminal and ground as short as possible.

WARNING: NEVER connect the [GND]

terminal to a gas or electric pipe, since the connectioncould cause an explosion or electric shock.

• Antenna connection

For radio communications, the antenna is of critical importance,along with output power and sensitivity. Selectantenna(s), such as a well-matched 50  antenna, and feedline. 1.5:1 or better of Voltage Standing Wave Ratio (VSWR) is recommended for your desired band. Of course, the transmission line should be a coaxial cable.

CAUTION: Protect your transceiver from lightning by using a lightning arrestor.

Antenna SWR

Each antenna is tuned for a specified frequency range and SWR may be increased out-of-range. When the SWR is higher than approx. 2.0:1, the transceiver's power drops to protect the final transistor.In this case, an antenna tuner is useful to match the transceiver and antenna. Low SWR allows full power for transmitting even when using the antenna tuner. The IC-718 has an SWR meter to monitor the antenna SWR continuously.

• Power supply connections

Use an optional PS-85 DC POWER SUPPLY when operatingthe IC-718 with AC power. Refer to the diagrams below.

CAUTION: Before connecting the DC cable, check the following important items. Make sure:

• The [POWER] switch is OFF.

• Output voltage of the power source is 12-15 V when you use a non-lcom power supply.

• DC power cable polarity is correct.

Red	: positive  terminal
	Black	: negative G terminal