- 1 -

Introduction:

The AT-11MP is a full featured auto or semi automatic antenna tuner designed for

HF (1.8 to 30 MHz) transceivers using between 5 to 150 watts. The tuner uses a switched "L"

configuration with 256 capacitor, 256 inductor and Hi/Lo-Z settings to provide over one hundred and

thirty thousand tuning combinations. The tuning range is 0 to 2700 pF and 0 to 20 uH. The "L"

network works great with just about any coax fed antenna (dipole, vertical, beam, etc). Users with

long wires can install a balun between the tuner and the antenna. Tuning time is between 0.1 and

6.0 seconds with the average being about 2.5 seconds.

A cross reading dual needle meter provide an indication of SWR while RF is present and DC power

is on. Since the power is actually calculated in the processor, the meter will not function with the

power off.

Operation:

Turning on the unit with the front panel Power switch will initialize the microprocessor.

The red LED will light and both meter needles will bounce once to indicate the power up process

was initiated. Turning the unit off by the front panel Power switch places the tuner into Standby.

During standby, the tuning section is in bypass. Turning the unit back on will set the tuning section

to the settings acquired before being in Standby.

The Auto/Semi switch controls how the tuning process will be envoked. In Auto mode (Auto/Semi

toggle switch in Auto), the tuner will seek a 1.5 match (or better) anytime the SWR rises above 3.0.

In Semi mode (Auto/Semi toggle switch in Semi), the tuner will seek a match only when the Tune

push button is pressed. Both modes require that more than 5 watts of RF power be present. If the

Tune button is pressed and there is less than 5 watts of power, both meter needles will bounce

once to indicate that there was not enough RF to tune properly.

The Capacitor and Inductor Up and Down switches are used for fine tuning the inductors and

capacitors and can be used in either, Auto or Semi mode. Pressing either of the Up switches will

increment that tuning section until it reaches the maximum range. When the maximum value is

reached, both meter needles will bounce. Conversely, pressing either of the Down switches will

decrement that tuning section until it reaches the minimum range. When the maximum value is

reached, both meter needles will bounce.

- 2 -

Besides starting the tuning process, the Tune push button has some other functions. Pressing the

Tune button and Cap Up switch at the same time (with no RF applied) will cause the Hi/Lo-Z relay

to move to the Hi-Z position. The forward power meter needle will bounce to indicate the change.

Pressing the Tune button and Cap Down switch at the same time (with no RF present) will cause

the Hi/Lo-Z relay to move to the Lo-Z position. The reverse power meter needle will bounce to

indicate the change.

Pressing both Down switches at the same time will reset the processor and place all relays in a

bypass position.

Pressing the Tune push button and the Ind Up button will turn on the audio feedback feature.

Pressing the Tune push button and the Ind Down button will turn off the audio. When an eight-ohm

speaker is connected to J4 and audio is turned on, audio feedback will provide an indication of the

SWR level. The feedback is a series of one to five beeps where one beep is an indication of SWR

below 1.5, two indicates SWR between 1.5 and 2.0, three indicates SWR between 2.0 and 2.5, four

indicates SWR between 2.5 and 3.0, and five indicates SWR above 3.

The back panel has SO-239s for connecting UHF type connectors to the antenna and transmitter. A

coaxial power jack is used to provide DC power to the tuner. The jack mates with a 5.5 OD, 2.5 id

coaxial power plug. Between 11 and 15 volts DC can be used. The power supply must be able to

provide up to 0.5 amps.

On the back panel is the IC-706 interface. When connected to an IC-706, it enables the “Tune”

button on the 706. Pressing the Tune button on the 706 will cause the 706 to temporarily switch

modes to CW, transmit 10 watts and start the AT-11MP tuning cycle. Once the tuning cycle is

complete the AT-11MP will signal the 706 that the tuning is complete and the 706 will return to the

previous mode and power settings.

Also on the back panel is the connection to the AT-11 remote unit. The remote allows for control of

most of the front panel functions. The remote consists of the auto/semi switch, the cap and inductor

up and down switches, the Tune push button, the green 1.5 swr LED and the Tuning in progress

LED. The remote connects to the AT-11MP via a 9-conductor shielded cable with female DB-9

connectors on each end. The remote is supplied with a 6 or 10 foot cable. The user may provide

other cable lengths.

- 3 -

When the remote is connected to the AT-11MP, all front panel functions will work in parallel. For

example, the user may start the tuning cycle by pressing the Tune push button on the front panel of

the AT-11MP or the remote panel. In the case of the auto/semi switch, if either switch is in the auto

position, the tuner will be in the auto mode.

Notes:

Most tuning situations with the AT-11MP is very straightforward. Simply transmit a

continuous carrier and press the tune button. The tuner will find the best match and stop tuning.

In either mode (Auto or Semi), if power (either RF or +12) is removed after the tuning cycle starts,

but before it finishes, the tuning will stop. The resultant tune will be undetermined; it may or may not

be a match.

In some extreme tuning cases, the power needle may waver while transmitting a continuous carrier.

This indicates that the tuner is near the tuning limits. You may be able to find a slightly better tune

with the manual adjustment toggle switches.

Performance:

The actual performance from the small package will surprise you. It really tunes a lot

of antennas to a lot of places! Here are some of the actual test results.

A 40-meter dipole (at 30 feet) would tune just about anywhere from 3.1 to 30 MHz! We had some

problems at 19 and 28 MHz finding a 1.5 match. The AT-11MP usually found a 2.0, and then we

had to use the manual switches to get below 1.5.

Next, an Antron-99 (at 40 feet) would again tune just about anywhere from 30 to 5 MHz. The auto

mode worked great the whole time. We just dialed down the band and the AT-11MP would kick in

whenever the SWR went over 3.0.

Then we tried out the unit on a 3-element tri-band (20,15,10) at 70 feet. It would tune any of the

ham bands (including WARC) except 160 and 80. We had some problems finding a match around

27.200 MHz, but got around it by moving to 27.000, letting the AT-11MP find a match, then moving

back to 27.200.

The 80-meter inverted Vee was next. It tuned everything from 3.1 to 30 MHz. We were hoping that

it would tune all of 160 meters, but it only found good matches from 1.900 to 1.800 MHz.

We've tried many other antennas with similar good results. There may be a place or two that your

antenna (dipole, inverted-Vee, vertical, beam, etc.) won't tune. Also, the farther away from

resonance you try to tune, the harder time the tuner will have. The AT-11MP will tune a 10 meter

vertical to 80 meters, but your performance will not be that great (you can't get something for

nothing).

For balanced lines and random wires, you may get better performance by using a 4 to 1 or 6 to 1

balun between the antenna and tuner.

We used the Autek RF-1 analyzer to give us more information about how well the AT-11MP was

working. We found that it would consistently tune impedances from about 6 ohms to about 850

ohms. This corresponds to an SWR of about 10:1.

The SWR bandwidth (usable bandwidth of 1.5 SWR without retuning) averaged about 200 kHz. On

the lower frequencies it was smaller (about 75 kHz on 80 meters) and on the higher frequencies it

was larger (about 400 kHz on 10), no surprises here.

- 4 -

Tech Support: Telephone technical support at 410-586-2177 is available most days from 8 am to

9pm Eastern Standard Time. Replies by FAX (410-586-8475) are welcome; e-mail

(ldg@ldgelectronics.com) is also answered on a daily basis.

Last Resort:

As a last resort only, LDG Electronics will attempt to repair any problems. We have a

flat fee of $50 plus parts to repair a AT-11MP (most resistors and capacitors are included in that

fee). The 68HC11 chip is the most expensive at $20. Relays are $5 each. The 78L05 is $1.

We will not attempt to repair any unit that has been soldered with acid core. We reserve the right to

refuse repair due to excessive problems or damage due to construction.

Before any unit is sent to us, you must first call to get return authorization. All units sent back must

be prepaid, either by check, money order or Credit Card unless other arrangements are made.

Package the unit carefully and keep in mind that we will use your packaging to return the unit back

to you. Include a description of what problem you are having along with your name, address and a

phone number that you can be reached at in case we have questions. Repairs average about 2 to 4

weeks, depending on the particular problem.

If you have an idea of how the unit can be made better (in software or hardware), please send a

description of your upgrade. If we use it for the AT-11MP, we'll send you a free upgrade. Future

upgrades will be available for about $10 with 68HC11 chip trade in. If you purchased the kit from

LDG, we will notify you when future upgrades and modifications are available through the AT-11MP

Newsletter. If you purchased the AT-11MP through a distributor and would like to be added to our

mailing list, just drop us a note by mail, fax or e-mail.

Feedback: We encourage everyone who uses the AT-11MP to drop us a note (card, letter or e-mail

preferred) to let us know how well it works for you. We're also always on the lookout for

photographs of the AT-11MP in use. We frequently place pictures that we receive into the LDG

Newsletter or on our Web site (www.ldgelectronics.com).